# Master Specification — India-Scale End-to-End Verifiable Voting (Wheel + One-Slot + Reusable Module)

Version: Spec-WheelSlot-Reusable-v1.4.0 — 2025-12-28
License: CC BY 4.0
Authorship (page-1 header only): Madhusudan Gopanna (primary); GPT-5 Thinking (co-author)

## 0) Goal

Design and deploy a national-scale, low-cost, end-to-end verifiable voting system that:

- Preserves ballot secrecy and receipt-freeness (no voter can prove how they voted, not even themselves).

- Enables voter-level inclusion verification (each voter can confirm their ballot is in the final tally).

- Guarantees one person, one counted vote with public, append-only evidence.

- Operates offline-tolerant, power-frugal, and is manufacturable at low cost for low- and middle-income countries.

- Resists manipulation by powerful incumbents via protocol, law, and real-time transparency tripwires.

## 1) Scope

- Covers: Polling-place hardware (reusable voter module, one-slot wheel interface), eligibility and authorization flow, cryptographic commitments, public bulletin board (permissioned consortium ledger), transparency telemetry, audits, governance, accessibility, operations, red-team testing, and acceptance metrics.

- Excludes: Remote/Internet voting; permanent personal devices; party-specific canvassing tools; postal ballot logistics (handled under same verifier rules but outside this spec).

### 1A) Provenance & Authorship

- Primary author & systems architect: Madhusudan Gopanna (global applicability), 2024–2025.

- Technical co-author (collaborative AI): GPT-5 Thinking — drafting, pressure-testing, cost modeling, editorial clarity.

- Editorial stance: Non-partisan, pro-democracy. Designed for adoption by any lawful election authority worldwide (not India-only).

- Versioning: VerifiableVoting-v1.3.2-2025-12-28. Change log in Appendix AB.

- License: Creative Commons CC BY 4.0 — reuse with attribution (“Gopanna — primary; GPT-5 Thinking — co-author”).

- Attribution presentation (PDF builds): Page-1 header only (cover is neutral). No authorship on placards.

## 2) Adversary Model

- Capabilities: Control of parts of bureaucracy/procurement, influence over media narratives, ability to starve capacity or delay logistics, access to vendors, and policy levers for quiet rule changes.

- Constraints: Must maintain plausible deniability; cannot openly eliminate elections; faces statutory tripwires and multi-party oversight.

## 3) System Overview (Narrative)

- Voter is verified at check-in and issued a one-time authorization (short-lived cryptographic token or printed slip with QR/serial) bound to that station’s event.

- Voter picks any reusable module from a pooled tray.

- At the machine, voter turns module to VOTE, inserts into the single slot. The voter rotates a knurled wheel until the desired candidate is framed in a shrouded bracket, then confirms (2-second hold).

- Inside the module, a rotating minute-key (SE) and a window nonce are used to compute voterhash; the machine then computes a votehash = H(voterhash, selection, device-only secret, window). The cast runs in fixed 60 s.

- The voter receives a receipt that contains the pair {voterhash, votehash} (no choice revealed, no proof of choice possible).

Sample receipt for illustrative purpose only – not real hash:

Inputs (UTF-8, ‘|’ separator):

- minute_key = MK_EXAMPLE_0001

- device_secret= DEVSEED_ABC123

- window_nonce = WIN_2025-11-12T10:05Z

- selection_id = CID-07

Election: IN-202X-LS-Phase3

Booth: TN-123-045

Batch Root: 5d3879de4ca2463fe8b6600b172efef735429b4d439df02afc6d33d64cb952e2

voterhash: db39cf612ba0023a342053d5394ee6e4c9812b3f805228a4debfe375d7ac9695

votehash: 13c0ee0ad55b9d98a2065a3c0018c8d5b7943ff9d5ac83246aca5b49373e231e

Use any verifier to check presence on the public ledger.

Inclusion Receipt (illustrative):

QR code would be generated with the following JSON:

{"ver":1,"batchRootShort":"5d3879de4ca2463fe8b6600b172efef735429b4d439df02afc6d33d64cb952e2","electionId":"IN-202X-LS-Phase3","boothId":"TN-123-045","voterhash":"db39cf612ba0023a342053d5394ee6e4c9812b3f805228a4debfe375d7ac9695","votehash":"13c0ee0ad55b9d98a2065a3c0018c8d5b7943ff9d5ac83246aca5b49373e231e"}

- The module resets to LOCK, forgets everything, and returns to the tray. The authorization is consumed (at most one accepted cast per authorization).

- The public ledger publishes batches (per §8/§8.3) containing: per-candidate counts, a set of voters:{voterhash…}, a set of votes:{votehash…}, a VRF proof of timing, and a Merkle root chaining to the previous batch. Voters later check inclusion via either hash in their receipt.

## 4) Security Properties (Must-Hold Invariants)

- SP-1 Secrecy / receipt-freeness: No artifact combination enables a voter to prove their choice.

- SP-2 Inclusion: Each voter can verify that their ballot is included (presence of either receipt value on the ledger).

- SP-3 Uniqueness: The ledger accepts ≤1 counted cast per valid authorization; duplicates/voids are visible and auditable.

- SP-4 Eligibility separation: The entity that verifies eligibility cannot link a person to a posted vote entry.

- SP-5 Append-only public record: Any attempt to prune/alter posted entries is detectable by independent replicas.

- SP-6 Constant-time casting & anti-side-channels: Choice does not affect timing, power, EM signature, audio/LED cues, or operator workflows.

- SP-7 Accessibility parity: Assisted voting preserves SP-1..6; assisted voters are not deanonymized by SKU or workflow.

- SP-8 VRF verifiability: Batch timing is chosen by a public VRF; anyone can recompute and detect operator discretion.

- SP-9 Booth-binding auditability: Every module re-bind is publicly posted; bound modules cannot cast outside the booth cert.

## 5) Polling-Place Hardware

### 5.1 Reusable Voter Module (Core, No Haptics)

- Mechanical: Palmable sealed unit; 2-position turnkey: LOCK → VOTE (clockwise). IP54 shell; tamper mesh; epoxy potting.

- Power: Supercap + 200–300 mAh cell; rack charging via pogo/USB-C; tri-dot LED for battery. (No PV or hand-crank on default module.)

- Compute: Low-power MCU; secure element (SE/HSM) with monotonic counter; RTC.

- I/O: Piezo beeper; 3 LEDs (status). No NFC; no APP/user diagnostics mode.

- Booth binding: Module is cryptographically bound to a specific booth ID and its registered machines; it only operates with those machines.

- Recalibration (re-binding): Requires an official provisioning device; upon re-bind, the module enters a 1-week lock/quarantine and emits a Rebind Event to the public audit log.

- Modes:

  - LOCK (default): all LEDs off; cannot cast.

  - VOTE: 10-second pre-insert window (LED chase); on insert → CAST 60 s constant-time; success → 1-second ACK then LOCK; error → purge + error tone + LOCK.

- Service/attestation: Diagnostics/attestation only via the provisioning jig under dual-control at depot; not in-booth and not via wireless.

### 5.2 Accessibility SKUs (identical externals)

- Audio SKU: Headphone jack; voice prompts; tactile confirm ring.

- Visual-Plus SKU: High-contrast LED bar near bracket; big-font mirrored text.

- Haptic SKU (deafblind): Sealed LRA; short buzz per detent; two buzzes on align; long on confirm. Duty-cycle limited.

- Off-Grid Power SKU (regional): Adds PV trickle panel and foldable hand-crank/dynamo; otherwise, identical firmware/exterior.

### 5.3 Wheel + One-Slot Voting Interface (Retrofit)

- Wheel: 60–80 mm knurled; uniform detents (0.25–0.35 N·m); encoder; neutral blank home index on reset.

- Slot & Hood: Single aperture with deep hood and side baffles; identical behavior for all candidates; anti-jam chamfer; sealed faceplate.

- Confirm: Sealed button; 2-second hold to commit; identical cues for all candidates.

- Parity: No per-candidate LEDs, tones, or timing variation; dummy loads equalize EM/power signatures.

- Booth binding: Machine carries a booth certificate; accepts casts only from modules bound to that booth.

### 5.4 Charging & Disinfection Rack

- Slots: One per module (N = 3 × M_v). Slots mechanically mimic the machine’s slot but only provide charging + UV-C disinfection; no casting state is possible in the rack.

- Electrical: Pogo/USB-C charging; per-slot current limit and health telemetry (voltage, temperature, cycle count).

- Power & health: Charge-state LEDs; cooling as required; rack reports fleet health to the station console.

- Security: Data-diode style ports (power/health only); no firmware writes; lockable door; inventory reconciliation at open/close.

### 5.5 Voter-Facing Test Mode (Spot-Check)

- Purpose: Allow any voter to spot-check that a module and machine are functioning correctly without affecting election data or revealing candidate mappings.

- Activation: Poll worker toggles TEST on the machine (keyswitch or sealed menu). Panel shows "TEST"; machine will not accept CAST.

- Sample wheel overlay: Removable Test Wheel disc printed with random letters/symbols (not candidate names/numbers). Mapping is randomized daily and posted on a placard for test only.

- Flow:

  - Voter takes a bound module in LOCK; worker sets machine to TEST.

  - Voter turns module to VOTE, inserts, rotates wheel to the prompted letter/symbol, and presses Confirm.

  - Machine displays "You selected: <symbol>".

- Timing & privacy: No voterhash/votehash is ever generated; nothing posts to the ledger. Test duration is constant-time equal to CAST (60 s) and uses identical cues.

- Auditing: TEST increments a public Test counter on the station dashboard; no per-module/per-voter IDs.

- Abort & safety: Exiting TEST requires keyswitch + dual-officer confirmation. Any mismatch quarantines the machine and prints a TEST FAIL incident code.

## 6) Eligibility & Authorization

- Check-in: Officer validates voter; issues a one-time authorization bound to that station’s event.

- Consumption: Machine accepts at most one CAST per valid authorization; duplicates visibly rejected and logged.

- Accounting: Station dashboard shows authorizations issued, casts accepted, voids with reason; totals reconcile at close.

## 7) Cryptographic Commitments (Abstract)

- Rotating minute-key: Derived inside SE from secret seed + time; non-exportable.

- Voterhash: Computed inside the module from the minute-key, a device-only secret and a window nonce; not linkable to identity or choice.

- Votehash: H(voterhash, selection, device-only secret, window); does not reveal choice without the secret.

- Receipt: Voter receives {voterhash, votehash}; sufficient to prove inclusion, insufficient to prove choice.

- Batch publication: Ledger publishes per-candidate counts, sets voters:{voterhash…} and votes:{votehash…} (order randomized; no per-ballot timestamps), VRF proof of timing (§8.3), and Merkle chaining to the prior batch.

## 8) Consortium Ledger (Public Bulletin Board)

- Validators: Election Commission, recognized opposition parties, apex court IT cell, national universities. Appends require threshold signatures.

- Snapshots: Hourly state snapshots mirrored to public archives; divergence alarms within 5 minutes.

- Openness: APIs and snapshots are public; verifier source is open.

- Batch record (conceptual):

{

electionId, constituencyId, boothId, batchId, machineId,

counts: { candidateId -> N },

voters: { voterhash, ... },

votes:  { votehash,  ... },

prevRoot, root, vrfProof, beaconEpoch

}

## 9) Operational Telemetry & Tripwires

- Live station metrics: uptime, queue length bands, modules in service, attestation pass-rate, modules bound vs present, authorizations vs casts vs voids.

- Tripwires: Z-score outliers auto-open incident tickets; breaches of wait-time SLO trigger surge redeployment or paper fallback.

- Privacy: Telemetry carries no candidate or voter identity; reported at 5–10-minute granularity.

## 10) Capacity Planning (Normative)

- Let T = voters assigned to booth; H = hours open; λ = T/(60H) votes/min required.

- Each module yields ~1 vote/min (fixed 60 s).

- Booth provisioning rule: If a booth has M_v voter machines, stock N = 3 × M_v reusable modules. Rationale: one casting, one being returned/checked, one on rack per machine.

- Typical India booth (T≈1,000, H≈11, M_v≈2): N = 6 modules suffices; urban peaks scale with M_v.

- Surge cache at sector depot: +1 × M_v sealed spares.

## 11) Privacy & Side-Channel Controls

- Constant-time CAST (60 s); batch posting per §8.3; no per-candidate signals.

- Shrouded wheel + bracket; no overhead line-of-sight of selection.

- No personal wireless interfaces on modules or machines.

- Accessibility parity: identical enclosures with no outward label across SKUs.

## 12) Attestation, Supply Chain & Tamper

- Measured boot: Firmware hashes on allow-list; SE holds attestation key; service-mode challenge/response only via sealed provisioning jig under dual-control.

- Mixed vendors, lot randomization: Prevent monoculture capture; distribution logs public.

- Tamper mesh: Opening zeroizes SE and raises a permanent visible flag; devices quarantined and logged.

- Booth binding certificates: Modules possess a Booth-Bind Cert signed by provisioning authority; machines verify this matches their Booth ID before CAST. Re-binding emits a signed Rebind Event to the ledger and enforces a 7-day lockout.

## 13) Availability & Chaos Engineering

- Brownout drills: PV + crank SKU + supercap sustain CAST in off-grid precincts; local cache persists until link resumes, then batch-commit.

- Surge redeployment: Pre-signed logistics to move spares within 30 minutes when SLO breached.

- Fallback: Paper contingency sealed and reconciled to ledger by RLA rules.

## 14) Governance & Law (Binding Requirements)

- In statute: machine-readable rolls; public diffs per revision; observer access; consortium ledger membership; open verifier; RLAs; publication SLAs for telemetry and snapshots; whistleblower protections; procurement transparency; VRF beacon source fixed in statute.

- Change control: Supermajority + cooling-off for any rule change affecting audits, telemetry, or proofs; civil injunction path within 48 hours.

## 15) Audits & Acceptance

- Pre-election: Red-team exercises (Appendix A) with public after-action reports.

- Election-day metrics: Attestation pass-rate ≥ 99.95%; 95th-percentile wait ≤ 20 min (urban) / 30 min (remote); zero unauthorized ledger rewrites; inclusion verification success ≥ 99.9% on first try.

- Post-election: RLAs and cryptographic verification; constituency-wise transparency packs (snapshots, verifier, how-to inclusion guide in major languages).

## 16) Cost & Deployment (Indicative)

- Module BOM (core): ~$10–$13 at national scale (PV/hand-crank removed from default).

- Off-Grid Power SKU: add ~$3 per unit (PV + crank) where required.

- Wheel retrofit: ~$15–$20 per machine.

- Station kit (normative): N = 3 × M_v modules + charging/UV rack + retrofit + spares, reusable for 5–7 years.

- National roll-out: For ~1.05M booths, hardware capex under pooled-reuse model scales with M_v; see Appendix E for scenarios.

## 17) Rollout Plan (Phased)

- Pilot (≤1%): mixed urban/rural; public observers; publish full data.

- Scale-up (10–25%): incorporate after-action fixes; independent audit sign-off.

- Nationwide: statutory tripwires live; transparency dashboards public; bug bounty on verifiers.

## 18) VRF-Governed Sliding-Window Cadence (SWC)

Definition. For each batch, a verifiable randomness function (VRF) seeded with the prior batch root and a public randomness beacon selects a step Δ ∈ {−2, −1, 0, +1, +2} to shift a center index over 5-minute bins, forming a ±2 sliding window. Using the same seed, the system deterministically selects the batch interval R from that window. Batches close on time, size, idle, or end-of-poll—whichever occurs first. Operators have zero discretion. Observers recompute Δ, the window, and R from the published VRF proof in the batch header.

Rationale. SWC is unpredictable to adversaries, reproducible to auditors, and resistant to operator timing games. It reduces timing-correlation fingerprints while preserving bounded latency.

Requirements.

- Publish the VRF proof and beacon epoch in the batch header.

- All machines in a booth share the same R for a given batch.

- No per-ballot timestamps; randomize order of votes and voters sets.

- Optional publication jitter ±(30–60 s) after batch close.

## 19) Designated Paper-Trail Booths (Eco-Minimal)

Scope. Only booths pre-earmarked for paper trail produce physical artifacts; all others remain paper-quiet.

When to print. At each batch close (per §8.3 SWC) in an earmarked booth.

What to print (Batch Anchor Slip).

- Batch Merkle Root (full) and short ID (QR optional).

- Prev Root (short) to show chaining.

- Counts per candidate for this batch.

- Header: Election ID, Constituency ID, Booth ID, Voter Machine ID, Batch ID, Window Label (SWC).

- Signatures: two officers + one observer.

What never appears on paper. No votehash list, no voterhash list, no per-ballot timestamps, no mapping from votehash → candidate.

Chain of custody. Affix slips to a Daily Audit Sheet or store sequentially in a sealed envelope; record slip count in the booth log.

Public verification. Anyone can scan or enter the printed root to confirm it matches the public ledger; candidate counts match posted proofs. Slips are ancillary anchors, not primary records.

## 20) Representative Diagrams (stroke-only, black & white)

Fig. 1 - Two-position key (LOCK→VOTE); sealed secure element; edge contacts; no NFC/BLE; booth-binding.

Fig. 2 – Wheel can rotate either direction; wheel selects into bracket; single hooded slot; constant-time cast; confirm button.

Fig 3 – Pooled module rack with charge/UV; flow: issue → cast → return; modules are booth-bound.

# Appendices

## Appendix A — Red-Team Playbook (Public)

Goal: Attempt to violate secrecy, inclusion, append-only behavior, attestation, booth-binding, or VRF cadence without insider privileges, and with plausible operator constraints.

### A.1 Threat Matrix (examples)

- Timing games: Correlate per-voter cast time with posting time to infer choice.

- Batch tamper: Prune or reorder posted entries without breaking chain validation.

- Booth-binding bypass: Use a bound module at the wrong booth/machine.

- Attestation forgery: Boot unauthorized firmware or spoof SE attest.

- Side-channels: EM/audio/power differences that correlate with candidate selection.

- Receipt proving: Use receipt artifacts to prove choice to a third party.

- VRF manipulation: Predict/steer the sliding-window cadence (SWC) or suppress proofs.

### A.2 Test Cases (sample)

- Timing: Capture ingress camera timestamps vs batch publication times; show inference failure (AUC≤0.55).

- Batch integrity: Try removing one `votehash` locally; verifier must detect chain break via `prevRoot/root`.

- Binding: Module from Booth X fails CAST at Booth Y; ledger logs no CAST; local audit shows “binding mismatch”.

- Attestation: Swap MCU image; provisioning jig must reject SE quote; device quarantines.

- Side-channel: Measure current draw for 1000 casts across candidates; distributions indistinguishable (KS-test p>0.1).

- Receipt proving: Attempt to construct a provable link from receipt to candidate; must be impossible without internal secrets.

- VRF: Pre-compute R from public beacon only; must be infeasible without prior batch root.

### A.3 Success Criteria

- Zero successful secrecy or append-only breaks.

- Inclusion checks reproducible by any third party (≥99.9% success).

- Binding/attestation/VRF incidents auto-quarantine with logged evidence.

### A.4 Reporting

- Public after-action within 30 days: scenario, exploit path, mitigations, patches, verification artifacts.

## Appendix B — Glossary & Normative Language

- CAST: Fixed, constant-time vote casting window (nominally 60 s).

- Receipt-freeness: A voter cannot prove their choice to a third party.

- Booth-binding: Module/machine cryptographic binding to a specific booth.

- VRF: Verifiable Random Function; governs batch timing window and interval.

- SWC: Sliding-Window Cadence derived from VRF (±2 bins over 5-minute centers).

- RLA: Risk-Limiting Audit; statistical audit of reported outcomes.

- MUST/SHOULD/MAY: RFC-2119 normative keywords used throughout this spec.

## Appendix C — Verifier Test Vectors

### C.1 Example Receipt Payload (QR)

{"ver":1,"batchRoot":"5d3879de4ca2463fe8b6600b172efef735429b4d439df02afc6d33d64cb952e2","electionId":"IN-202X-LS-Phase3","boothId":"TN-123-045","voterhash":"db39cf612ba0023a342053d5394ee6e4c9812b3f805228a4debfe375d7ac9695","votehash":"13c0ee0ad55b9d98a2065a3c0018c8d5b7943ff9d5ac83246aca5b49373e231e"}

### C.2 Example Batch Record (conceptual JSON)

{

"electionId": "IN-202X-LS-Phase3",

"constituencyId": "TN-123",

"boothId": "TN-123-045",

"batchId": "TN-123-045-0007",

"machineId": "VM-A",

"counts": { "CID-01": 103, "CID-02": 97, "CID-07": 111 },

"voters": [

"db39cf612ba0023a342053d5394ee6e4c9812b3f805228a4debfe375d7ac9695",

"9a3b8e7c0b9f4d83e8e29b7f0d1bf3cd8083a7d9a2f2b3c4d5e6f7a8b9c0d1e2"

],

"votes": [

"13c0ee0ad55b9d98a2065a3c0018c8d5b7943ff9d5ac83246aca5b49373e231e",

"9f52b7f5b2f8d4a1f0c9e8d7c6b5a4f3e2d1c0b9a8f7e6d5c4b3a29181726354"

],

"vrfProof": "VRF-PROOF-BASE64",

"beaconEpoch": "2025-11-12T10:05:00Z",

"prevRoot": "79f0e8b3d8e6d1c0b9a8f7e6d5c4b3a29181726354c6d7e8f9a0b1c2d3e4f5a6",

"root": "5d3879de4ca2463fe8b6600b172efef735429b4d439df02afc6d33d64cb952e2"

}

### C.3 Verifier Cross-Checks

- sum(counts.values()) == len(votes)

- len(votes) == len(voters)

- root == Merkle(batch_body) where batch_body excludes root

- prevRoot links to previous batch’s root

- vrfProof validates (Δ,R) against prevRoot + beaconEpoch

## Appendix D — Receipt & QR Specification

### D.1 Canonical Receipt Payload (QR)

- Encoding: UTF-8 JSON (option: CBOR in future)

- Fields:

  - ver (uint) — schema version

  - batchRoot (64-hex) — full Merkle root of the batch

  - electionId (string)

  - boothId (string)

  - voterhash (64-hex)

  - votehash (64-hex)

### D.2 Printing Requirements

- ECC: Q (25%) or H (30%)

- Size: ≥ 25 mm square; high contrast; matte stock

- Quiet zone: ≥ 4 modules

### D.3 Human-Readable Lines (on paper)

- Truncate only in print preview (e.g., first/last 8 hex chars).

- Do not truncate in QR.

D.4 Privacy Constraints

- No per-ballot timestamps

- No operator/station PII beyond boothId already public

- No candidate identifier in receipt

## Appendix E — Cost Scenarios (Indicative)

| Scenario | M_v (machines /booth) | Modules per booth (N=3×M_v) | Core Module BOM | Off-Grid SKU delta | Wheel Retrofit / machine | Rack (per booth) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Base rural | 1 | 3 | $10–13 | +$3 | $15–20 | $40–60 | Low power; off-grid in 20% booths |
| Standard | 2 | 6 | $10–13 | +$3 | $15–20 | $60–90 | Typical India booth (T≈1k, H≈11) |
| Urban peak | 3 | 9 | $10–13 | +$3 | $15–20 | $80–120 | Add surge cache +3 modules/sector |

Reuse horizon: 5–7 years hardware; firmware OTA under measured-boot.
National roll-out: ~1.05M booths × scenario mix; procurement diversified across vendors.

## Appendix F — Accessibility Parity Checklist

- Timing parity: Assisted SKUs use identical CAST 60 s window.

- Cue parity: Audio/visual/haptic cues standardized (no candidate-specific signals).

- Enclosure parity: External appearance indistinguishable; masking caps for special ports.

- Path parity: No distinctive operator handling that could deanonymize.

- Attestation parity: Same SE/attestation; SKU flag not externally exposed.

- Documentation: Public guidance in major languages; tactile and audio instructions.

## Appendix G — Governance Text Pack (Model Clauses)

G1. Public Rolls & Diffs (statute):
“The Commission SHALL publish machine-readable electoral rolls and SHALL publish a public diff for every revision.”

G2. Consortium Ledger:
“A public, append-only bulletin board SHALL be maintained by a consortium of validators (Commission, recognized opposition parties, apex court IT cell, national universities) with threshold signatures.”

G3. VRF Beacon Source:
“The batch cadence SHALL be governed by a VRF seeded from the prior batch root and a public randomness beacon designated by statute.”

G4. Open Verifier & Snapshots:
“The reference verifier SHALL be open-source. Hourly snapshots SHALL be published with divergence alarms ≤ 5 minutes.”

G5. RLAs & Telemetry:
“Risk-limiting audits SHALL be conducted. Station telemetry SHALL be published at 5–10 minute granularity without PII.”

G6. Change Control:
“Changes to audit/telemetry/proof rules REQUIRE supermajority approval and a 30-day cooling-off period with injunctive relief available within 48 hours.”

## Appendix H — Operational Telemetry Schema (Public)

### H.1 Station Metrics (5–10 min)

{

"electionId": "IN-202X-LS-Phase3",

"boothId": "TN-123-045",

"uptimeSec": 39240,

"queueBand": "0-5|5-15|15-30|>30",

"modulesPresent": 6,

"modulesBound": 6,

"castsAccepted": 742,

"authIssued": 760,

"voids": 8,

"attestationPassRate": 0.9996,

"testModeCount": 14,

"lastBatchRoot": "5d3879de4ca2463fe8b6600b172efef735429b4d439df02afc6d33d64cb952e2"

}

### H.2 Divergence/Tripwire Events

{

"type": "TRIPWIRE",

"boothId": "TN-123-045",

"metric": "queueBand",

"zscore": 3.2,

"threshold": 2.5,

"action": "SURGE_DEPLOYMENT"

}

## Appendix I — Provisioning, Attestation & Rebind

### I.1 Measured Boot & Attestation

- SE holds device keypair; MCU boots allow-listed hashes; attests (fwHash, seState, boothId) to provisioning jig.

- Station accepts CAST only after verifying attestation stamp within validity window.

### I.2 Booth Binding

- Module stores Booth-Bind Cert signed by authority; machine presents Booth ID; CAST proceeds only if match.

### I.3 Rebind Event & Quarantine

- Rebinding requires dual-control jig; emits on ledger:

{

"event": "REBINDBIND",

"moduleId": "MOD-XYZ-123",

"fromBooth": "TN-120-002",

"toBooth": "TN-123-045",

"when": "2025-11-01T09:00:00Z",

"sig": "BASE64-THRESHOLD-SIG"

}

- Module enters 7-day lockout before activation at new booth.

## Appendix J — Formal Threat Model (Summary)

### J.1 Adversary Capabilities

- Partial control of logistics/procurement/media; limited insider influence; access to vendors; policy levers for quiet rule changes.

### J.2 Constraints

- Must preserve plausible deniability; elections cannot be abolished; subject to public proofs and multi-party oversight.

### J.3 Assumptions & Trust Roots

- Mixed vendors; SE supply diversity; threshold-sig validator set; public randomness beacon integrity.

### J.4 Mitigation Mapping

- Secrecy: constant-time casting; receipt excludes choice; no per-ballot timestamps.

- Inclusion: dual-hash receipt; public sets {voters}, {votes}.

- Append-only: Merkle chaining prevRoot→root; hourly snapshots; third-party mirrors.

- Binding/Attest: Booth-Bind Certs; measured boot; public Rebind Events.

- VRF/SWC: Window and R from VRF(prevRoot, beacon); operator has zero discretion.

## Appendix K — Change Log & Citation

- v1.0 — 2025-11-09: Initial consolidated master spec; booth-binding; N=3×M_v provisioning; rack charging/UV; voter-facing Test Mode; English placards; cost tables; authorship section.

- v1.1.0 — 2025-11-09: Removed NFC & APP user mode; two-position key (LOCK→VOTE); updated privacy and attestation; updated module BOM; clarified Test Mode (LOCK only); editorial fixes.

- v1.2.0 — 2025-11-09: Added §8.3 SWC (VRF-governed cadence; ±2 window; zero operator discretion).

- v1.3.0 — 2025-11-09: Dual-hash receipt {voterhash, votehash}; explicit ledger sets (voters, votes) with randomized order; SP-8 (VRF verifiability) and SP-9 (booth-binding auditability); clarified Test Mode (no hashes, 60 s); batch record box in §8; governance adds statutory VRF beacon.

- v1.3.1 — 2025-11-12: Editorial sync; version & citation alignment; text-only PDF build.

- v1.4.0 – 2025-12-28: Additional details; improved diagrams; JSON and QR code for improved verification.

How to cite:

Gopanna, M. (primary), & GPT-5 Thinking (co-author). Master Specification — India-Scale End-to-End Verifiable Voting (Wheel + One-Slot + Reusable Module), v1.4.0, 2025-12-28. CC BY 4.0.